Checklist for survey of Ship Security Alert System

Date:

| M/V | Class No |
| --- | --- |
| Type of Ship | IMO No. |
| Office | Port |
| Company of ship | Maritime Security Auditor : |

Tick in the left box of following with satisfaction;

| 1. General |  |  |
| --- | --- | --- |
|  | 1-1 | Ship Security Alert System is complies with IMO Res.ΜSC. 136 (76) or Res. MSC 147(77) and/or IEC 60945. |
| 2. Power supply |  |  |
|  | 2-1 | In addition to the main power supply, an alternative source of power is available. |
|  | 2-2 | Alternative source of power is either emergency power supply, a storage battery charged with emergency power source, or independent battery of 6 hours or more. |
| 3. Operation |  |  |
|  | 3-1 | Ship Security Alert System is capable of being activated from the navigation bridge and in at least one other location. |
|  | 3-2 | Ship Security Alert Messages solely addresses to competent authorities designated by the Administration. |
|  | 3-3 | Ship Security Alert System activation points is designed so as to prevent the inadvertent initiation of the ship security alert. It is not necessary for the user to remove seals or to break any lid or cover in order to operate any control. |
|  | 3-4 | The activation points operates a radio system such transmission of the security alert does not require any adjustment of the radio system, i.e. tuning of channels, setting of modes or menu options. |
|  | 3-5 | Once activated, Ship Security Alert System continues to transmit the security alert until it is reset or deactivated. The interval between transmissions of the alerts is as short as possible. (e.g. between 15 minutes and one hour) |
|  | 3-6 | Operation of the activation point does not cause any alarm or indication to be raised on the ship nor alert other ships. |
|  | 3-7 | The operation of the ship security alert system does not impair the functionality of the GMDSS installation. |
| 4. Ship Security Alert |  |  |
|  | 4-1 | Transmission initiated by security alert system activation points includes a unique code/identifier1 (ship’s identity, e.g. call sign, IMO ship identification number or Maritime Mobile Service Identify (MMSI)) and ship’s security alert activation indication. |
|  | 4-2  N/A | Where Ship Security Alert System is installed before 1 July 2004, the transmission includes the ship identity and current position (latitude and longitude). |
|  | 4-3  N/A | Where Ship Security Alert System is installed on or after 1 July 2004, the transmission includes the ship identity and current position (latitude and longitude) associated with a date and time. |
|  | 4-4 | Ship Security Alert Messages (Ship’s identity, ship’s position and ship’s security alert activation indication) are generated automatically with no input from the operator other than security alert system activation. |
| 5. Installation |  |  |
|  | 5-1 | General inspection. |
|  | 5-2 | Wiring (power cable and antenna cable) has been permanently installed, according to the manufacturer’s instructions. |
|  | 5-3 | Operation test of SSAS, the latter being activated from all activation points. Transmission of the security alert to the CSO. |
| 6. Others |  |  |
|  | 6-1 | The Ship Security Alert System is capable of being tested. |
|  | 6-2 | Operation, maintenance and installation manual provided by the manufacturer are placed onboard. |
|  | 6-3 | The Master and the SSO are familiar with the operation of SSAS and, the resetting procedure, if any. |

______________                                                                    ___________________________

Radio Company Inspector	                                            Surveyor/Maritime Security auditor

(Name, signature)				                                (Name, signature)